#01044-Revised 7-99  Trousers, Boot, Male Gabardine
Page 1 of 13


Texas A&M Corps of Cadets Specifications

Trousers, Boot, Male, Gabardine

Description:
Silver tan gabardine material:  11 ½ oz. or 12 oz. 55% Dacron polyester, 45% Wool, two-ply warp and 2 ply filling:  Raeford #521-193 (Approx. 1 ½ yds. per pair)  Inserts-white indestructible.  




Use two buttons closing to the inside on waistband.  (Do not use hook closures at front waist belt-line.)

Waistband to be finished in an open manner (Browne outlet method) with a minimum of two and one half inch outlet showing no raw edges.  The waistband seam shall be pressed open.




Please refer to all photos and drawings.




Each pair requires lateral zippers on the outside of both pant legs, below knee to bottom of each leg.  Polyester double fold bra’s hem tape sewn on bottom of each pant leg.  Four pockets, one watch pocket, and white inside patches to both knees.  Elastic strap, 15” on each leg bottom.

Trousers shall be constructed with a “Browne outlet” as noted above and shall have a one inch inseam outlet in back panel from crotch to full length of inseam.




NO ELASTIC MATERIAL INSETS REQUIRED WITH ZIPPERS!

All interlining shall be poly/cotton and pocket lining shall be poly/cotton twill.

STITCHING TECHNIQUE FOR SECURING KNEE PATCHES


Double stitch the patch to the trouser leg.  There will be three sharp pivot points at the corners.  Back stitch about ¼ to 3/8 inch on either side of the corners, then proceed on the next side without breaking the thread.  Turning the garment during the course of this operation will make this possible.  The object is to get well secured patch at these friction points.  Continuous stitching with reinforced points will do this.  There is no need to back stitch the two slowly turning curves.  They can be stitched in one motion.  Please refer to the diagram for further clarification.

Seven (7) belt loops 1” wide for 2” belt.  Loops lined with pellon.  Watch pocket is inserted at right front pocket.  Side pockets have 4 “ opening across the top and 4” down the outseam.  Tan bone buttons with buttonhole at each upper corner of pockets.

See attached illustrations for measuring guides and general appearance.

[image: image1.png]


[image: image2.png]


[image: image3.png]


[image: image4.png]


[image: image5.png]


[image: image6.png]


[image: image7.png]


[image: image8.png]


[image: image9.png]


[image: image10.png]



[image: image11.png]







All waistbands are 2¼” wide from front then peaks in center back to 4”

Belt loops 2¼” in front and sides and peak center back 2 ¾” long. 

30 Extra Short:

Zipper and front seam from waist band measurement
9”



Back seam from waist band measurement
15.5”



Inside leg from crotch to knee
14.5”



Knee down to bottom of leg
9.25”



Mid-Calf
14”



Around bottom of leg
11”



Flare mid section (Front)
11”



30 Short:

Zipper and front seam from waist band measurement
9”



Back seam from waist band measurement
15.5”



Inside leg from crotch to knee
15”



Knee down to bottom of leg
9.25”



Mid-Calf
14”



Around bottom of leg
11”



Flare mid section (Front)
11”



30 Regular:

Zipper and front seam from waist band measurement
9.75”



Back seam from waist band measurement
16”



Inside leg from crotch to knee
16.5”



Knee down to bottom of leg
10.25”



Mid-Calf
14”



Around bottom of leg
11”



Flare mid section (Front)
11.5”



30 Long:

Zipper and front seam from waist band measurement
9.75”



Back seam from waist band measurement
16”



Inside leg from crotch to knee
17.5”



Knee down to bottom of leg
10.25”



Mid-Calf
14”



Around bottom of leg
11”



Flare mid section (Front)
11.5”



30 Extra Long:

Zipper and front seam from waist band measurement
9.75”



Back seam from waist band measurement
16”



Inside leg from crotch to knee
18.5”



Knee down to bottom of leg
10.25”



Mid-Calf
14”



Around bottom of leg
11”



Flare mid section (Front)
11.5”














Stitching Technique For Securing Knee Patches

Supplemental Drawing A


(Shown without pockets)





Band Peak





Other Belt Loops





Back Seam


from


Waistband





Back Seam





Band (Front)





Belt Loops


Front





Front Seam





Flare Area





White knee patches





Mid-Calf





View of Hi-back Construction





Pocket Stitching Detail





Photo 1





Example of Front Pocket Design/Stitching





Photo 2





Example of White Cloth “Knee Patch” on inside of leg.





No elastic inserts required with zippers.





Photo 3





View of zipper located on outside of each leg.





Photo 4





Notice:


Buttons Close on inside and are hidden from view.





Front Outside Detail





Photo 5





Front Inside Construction Detail





Photo 6





Rear Inside Construction Detail





Photo 7





BACKSTITCH





BACKSTITCH





BACKSTITCH





Supplemental Drawing B





Supplemental Drawing C





V-notch goes down to top of belt loop and bar tacked.





Belt loops lined with pellon and sewn under bottom of waistband.





Note:  Topstitching, flaps are self-fabric lined.








Heavy duty 9 ½ inch brass zipper used











B000008-Item 11


